Demo 1 - Fab - COIL - Montage Bobine - ENG

Date; Version; Created by; Approved by; Notes. 16/02/10; 1.0.1; Arnaud FREREJEAN; Création. 20/12/12; 1.0.2; Arnaud FREREJEAN. 10/09/13; 1.0.3; Arnaud FREREJEAN; MAJ Résumés. 10/11/16; 1.0.4; Arnaud FREREJEAN; English Version. 08/07/20; 1.0.5; Arnaud FREREJEAN; Regenerate. Who is concerned ? Workers that have to move coils What is it ? Moving coils instructions Where ? Workshop 64/58/94/91/54

Workshop

Tools

Operation 1 : Strapping

| Basic operations | Security | Selfchecking | Instruction | Gestual |
| --- | --- | --- | --- | --- |
| 1- Prepare Tools |  |  |  | O |
| 2- Strapping | compulsory gloves |  |  |  |
| 3- Take off the stick |  |  |  | O |
| 4- Pick up the strap |  |  |  |  |
| 5- Fasten to the winch |  | Check the link Check the quality of the link |  |  |
| 6- Shift the coil |  |  | Pull on the strap and shift the coil |  |

Instructions ? Strapping the coil Equipments ? Strap and Stick. The stick is used to pass the strap beneath the coil

Operation 2 : Lift and turn the coil

| Basic operations | Security | Selfchecking | Instruction | Gestual |
| --- | --- | --- | --- | --- |
| 1- The coil is shifted | Impacts risk, hazardious motion | The edge of the coil MUST be ON the next coil |  |  |
| 2- Lift the coil |  |  |  |  |
| 3- Turn the coil |  |  | Turn the tongue towards the machine |  |
| 4- Check the position |  | O |  |  |

Instruction ? Lifting and moving the coil

Point of particular attention 1 : Check the attachment of the hook with the strap

Point of particular attention 2 : Coil stability

Stabilize the coil to avoid - hazardious motion - personal or material demages while lifting

Point of particular attention 3 : Turn the coil (Tongue) towards the workstation

Turn the tongue towards the machine